# NeoMOM

“An amateur radio antenna prediction code”

## John Shaeffer

#### KZ4TS, NFARL

### 2026

Comments on modeling.  From Lee

Abstract

NeoMoM is a wire-antenna modeling suite designed for amateur radio operators and antenna engineers. It predicts input impedances, SWR, gain and radiation patterns using a full-wave electromagnetic solver based on Maxwell’s equations.

System Overview

The suite consists of three integrated applications that together support the complete antenna modeling workflow:

- neomom_input — Antenna geometry creation and editing

- neomom — Computational Method of Moments (MoM) engine

- neomom_plot — Visualization and interactive pattern analysis with geographic map overlay

Key Features

- Open-source design intended for community use, extension, and experimentation

- Modern Python-based graphical user interfaces for input and visualization

- Direct import of MMANA-GAL (*.maa) files with minor editing for excitation

- Export capability for NEC5 and MMANA-GAL input formats ready for further editing

Design Foundations

NeoMoM builds on the legacy of tools such as NEC, EZNEC, MiniNEC, and MMANA-GAL while introducing:

- A more rigorous computational framework

- A modern object-oriented architecture

- Improved usability through graphical interfaces

Advances in computing—such as increased memory, multi-core processors, and modern programming techniques—enable higher-performance simulations using object-oriented Fortran and optimized numerical libraries.  AI now allows development of modern GUI applications.

Core Concepts

Node-Based Geometry Definition

Antenna structures are defined using a list of tagged spatial nodes (x, y, z coordinates). From these nodes:

- Wire elements are constructed as tagged polylines

- Excitation is defined at a specific wire and node

- Geometry can be created either via GUI or text-based input

Full-Sphere Radiation Modeling

Neomom computes radiation fields (Eθ and Eφ) over the entire sphere. This global dataset enables:

- Arbitrary azimuth and elevation pattern cuts

- Full 3D radiation visualization

- Geographic map-based pattern overlays

Extensible GUI Framework

The input and visualization interfaces are implemented in Python using AI-assisted development. This approach encourages:

- Custom GUI development by users

- Experimentation and adaptation within the amateur radio community

System Components

Neomom_input

- Creates and edits *.nml input files

- Provides real-time 3D visualization of antenna geometry as nodes, wires, and excitations are defined.

- Supports:

  - Global parameters (frequency, ground type, title)

  - Node coordinate input (user-defined units)

  - Wire and excitation definitions

- Reads existing .nml and MMANA-GAL (.maa) files

- Exports NEC and MMANA-GAL *.nec and *.maa files which, after editing, can be used in NEC5 (EZ-NEC) and MMANA-GAL.

Neomom (Computational Engine)

- Full-wave Maxwell solver using the Method of Moments

- Triangle basis functions with Galerkin formulation

- Symmetric system matrix

- Input: Fortran namelist (*.nml)

- Output: Full-sphere complex Eθ and Eφ fields in *.csv format

Neomom_plot

- Input: Sphere values of Eθ and Eφ over full or half sphere, [θ,φ], data as output by neomom

- Visualization options:

  - 2D azimuth cuts (user-selected elevation)

  - 2D elevation cuts (user-selected azimuth)

  - 3D radiation patterns with ground plane

  - Top-down geographic view aligned to compass directions

- Advanced features:

  - Radiation pattern overlay on world, CONUS, or regional maps

  - Multiple map projections (azimuthal equidistant, Lambert conformal, plate carrée)

  - Adjustable orientation relative to geographic north

  - Scaling options: ARRL, dBi, dBpeak, or linear gain (Eθ, Eφ, total)

# neomom_input

# neomom_plot

# Validation

The following tables show input impedances and gain computed with neomom, NEC5, and mmana-gal for:

- Half wave dipole, center fed

- Folded dipole, center fed

- End fed half wave dipole

- Quarter wave vertical at and near ground, z = 0

- Disk cone VHF array of top hat radials and sloped cone radials

- Yagi, 6m, six element array

- Delta loop sized for 40 m.  Computed results for 40 and 10 m

- J Pole, 2m, above real ground

Dipole, folded dipole, and EFHW results are for free-space, perfect PEC ground, and real ground.

## Half wave dipole, centre fed

L = 20 m, f = 7.3 MHz, wire radius = 1 mm

Free Space, Perfect Ground, Real Ground

| FREE SPACE |  |  |  |
| --- | --- | --- | --- |
|  | Zin (ohms) | Gain (max) dB |  |
| Neomom | 71.9 – j 0.12 | 2.14 |  |
| Nec 5 | 71.6 - j 5.41 | 2.14 | GE = 0, GN = -1 |
| Mmana-gal | 74.5 + j 2.05 | 1.99 |  |

| 10 m above perfect ground |  |  |  |
| --- | --- | --- | --- |
|  | Zin (ohms) | Gain (max) dB |  |
| Neomom | 84.5 + j 30.5 | 7.58 |  |
| Nec 5 | 84.2 + j 25.0 | 7.58 | GE = 1, GN = 1 |
| Mmana-gal | 74.1 + j 14.7 | 8.02 |  |

| 5 m above perfect ground |  |  |  |
| --- | --- | --- | --- |
|  | Zin (ohms) | Gain (max) dB |  |
| Neomom | 31.1 + j 29.3 | 8.72 |  |
| Nec 5 | 31.6 + j 23.7 | 8.72 | GE = 1, GN = 1 |
| Mmana-gal | 59.6 + j 2.6 | 5.0 |  |

| 10 m above real ground (14.0,0.005) |  |  |  |
| --- | --- | --- | --- |
|  | Zin (ohms) | Gain (max) dB |  |
| Neomom | 83.1 + j 17.9 | 6.0 |  |
| Nec 5 | 82.5 + j 10.9 | 5.92 | GE = 1, GN = 2 |
| Mmana-gal | 84.7 + j 32.6 | 5.87 |  |

| 5 m above real ground (14.0,0.005) |  |  |  |
| --- | --- | --- | --- |
|  | Zin (ohms) | Gain (max) dB |  |
| Neomom | 48.4 + j 23.2 | 5.86 |  |
| Nec 5 | 55.6 + j 14.2 | 5.31 | GE = 1, GN = 2 |
| Mmana-gal | 31.2 + j 31.3 | 7.88 |  |

## Folded dipole, centre fed

L = 10.05 m, f = 14.05 MHz, wire radius = 1.5 mm, d = 0.2 m

fd20.maa file in the MMANA-GAL antenna library

Free Space, Perfect Ground, Real Ground

| FREE SPACE |  |  |  |
| --- | --- | --- | --- |
|  | Zin (ohms) | Gain (max) dB |  |
| Neomom | 285 – j 0.74 | 2.19 |  |
| Nec 5 | 286 – j 10.4 | 2.14 | GE = 0, GN = -1 |
| Mmana-gal | 286 + j 4.00 | 2.13 |  |

| 5 m above PEC |  |  |  |
| --- | --- | --- | --- |
|  | Zin (ohms) | Gain (max) dB |  |
| Neomom | 328 + j 131 | 7.71 |  |
| Nec 5 | 333 + j 121 | 7.71 | GE = 1, GN = 1 |
| Mmana-gal | 327 + j 136 | 7.7 |  |

| 5 m above real gnd |  |  |  |
| --- | --- | --- | --- |
|  | Zin (ohms) | Gain (max) dB |  |
| Neomom | 320 + j 74.2 | 5.87 |  |
| Nec 5 | 321 + j 56.8 | 5.85 | GE = 1, GN = 2 |
| Mmana-gal | 327 + j 136 | 5.83 |  |

## EFHF: End Fed Half Wave Dipole

L = 20 m, f = 7.3 MHz, wire radius = 1 mm

Free Space, Perfect Ground, Real Ground

| FREE SPACE |  |  |  |
| --- | --- | --- | --- |
|  | Zin (ohms) | Gain (max) dB |  |
| Neomom | 1453 – j 1109 | 2.15 |  |
| Nec 5 | 1484 – j 1296 | 2.15 | GE = 0, GN = -1 |
| Mmana-gal | 2036 – j 17481 | 2.17 |  |

| 10 m above PEC |  |  |  |
| --- | --- | --- | --- |
|  | Zin (ohms) | Gain (max) dB |  |
| Neomom | 2143 – j 1400 | 7.6 |  |
| Nec 5 | 2196 – j 1578 | 7.6 | GE = 1, GN = 1 |
| Mmana-gal | 1612 - j 18123 | 7.6 |  |

| 10 m above real gnd |  |  |  |
| --- | --- | --- | --- |
|  | Zin (ohms) | Gain (max) dB |  |
| Neomom | 1853 – j 1315 | 6.0 |  |
| Nec 5 | 1862 – j 1486 | 5.9 | GE = 1, GN = 2 |
| Mmana-gal | 1612 - j 18123 | 5.9 |  |

## Quarter Wave Vertical

L = 10 m, f = 7.3 MHz, wire radius = 1 mm

Perfect Ground, Real Ground

zH = 0.0 and 0.1 m

PEC Ground

| 0.0 m above PEC |  |  |  |
| --- | --- | --- | --- |
|  | Zin (ohms) | Gain (max) dB |  |
| Neomom | 27.2 – j 2334 | 5.18 |  |
| Nec 5 | 36.7 – j 2.5 | 5.15 | GE = 1, GN = 1 |
| Mmana-gal | 10.5 – j 17379 | 8.05 |  |

| 0.1 m above PEC |  |  |  |
| --- | --- | --- | --- |
|  | Zin (ohms) | Gain (max) dB |  |
| Neomom | 26.9 - j 2421 | 5.19 |  |
| Nec 5 | 28.0 - j 2781 | 5.19 | GE = 1, GN = 1 |
| Mmana-gal | 10.5 – j 17379 | 8.04 |  |

Real Ground

| 0.0 m above real gnd |  |  |  |
| --- | --- | --- | --- |
|  | Zin (ohms) | Gain (max) dB |  |
| Neomom | 47.0 – j 2429 | -2.5 |  |
| Nec 5 | 48.6 - j 0.701 | -1.25 | GE = 1, GN = 2 |
| Mmana-gal | 10.5 - j 17379 | 1.6 |  |

| 0.1 m above real gnd |  |  |  |
| --- | --- | --- | --- |
|  | Zin (ohms) | Gain (max) dB |  |
| Neomom | 38.4 – j 2478 | -1.67 |  |
| Nec 5 | 44.6 – j 2794 | -2.08 | GE = 1, GN = 2 |
| Mmana-gal | 10.5 - j 17379 | 1.62 |  |

## Disk Cone

8 Top Hat Radials, 8 Cone Slopers

Discon55-200.maa file in the MMANA-GAL antenna library

f = 145 MHz, wire radius = 5 mm

Free Space

| Free Space |  |  |  |
| --- | --- | --- | --- |
|  | Zin (ohms) | Gain (max) dB |  |
| Neomom | 137.5 + j 35 | 1.92 | θmax = 120 deg |
| Nec 5 | 135.6 + j 31.3 | 1.89 | GE = 0 |
| Mmana-gal | 134 + j 33.8 | 1.95 |  |

## Yagi, Six Element

6elYag.maa file in the MMANA-GAL antenna library

f = 50.5 MHz, wire radius = 7 mm

Free Space

| Free Space |  |  |  |
| --- | --- | --- | --- |
|  | Zin (ohms) | Gain (max) dB |  |
| Neomom | 34.5 – j 2.71 | 11 |  |
| Nec 5 | 35.3 – j 5.39 | 11 | GE = 0 |
| Mmana-gal | 34.3 – j 2.32 | 11 |  |

## Delta Loop

f = 7.25 MHz, wire radius = 1 mm

13 m above Real Ground

| 13m above gnd |  |  |  |
| --- | --- | --- | --- |
| 7.25 MHz | Zin (ohms) | Gain (max) dB |  |
| Neomom | 1749 - j 1582 | 5.77 |  |
| Nec 5 | 1945 – j 1563 | 5.79 |  |
| Mmana-gal | 1532 – j 1457 | 5.06 |  |

f = 29 MHz, wire radius = 1 mm

13 m above Real Ground

| 13m above gnd |  |  |  |
| --- | --- | --- | --- |
| 29 MHz | Zin (ohms) | Gain (max) dB |  |
| Neomom | 347 + j 614 | 13.4 |  |
| Nec 5 | 316 + j 568 | 13.5 |  |
| Mmana-gal | 367 + j 637 | 6.88 |  |

## J Pole, 2m

f = 145 MHz, wire radius = 1 mm

10 m above Real Ground

| 10m above gnd |  |  |  |
| --- | --- | --- | --- |
| 145 MHz | Zin (ohms) | Gain (max) dB |  |
| Neomom | 70.2 + j 86.7 | 6.38 |  |
| Nec 5 | 75.1 + j 89.0 | 6.56 |  |
| Mmana-gal | 72.7 + j 92.2 | 6.66 |  |

# J Pole, 2m, 0.05 λ Above Ground

This J Pole version is the correct gamma match feed

Different size and frequency than the version above

f = 146 MHz, wire radius = 1 mm

0.1 m (0.049 λ) above Real Ground

| 0.1 m above gnd |  |  |  |
| --- | --- | --- | --- |
| 146 MHz | Zin (ohms) | Gain (max) dB |  |
| Neomom | 57.6 – j 3.14 | 1.3 |  |
| Nec 5 | 60.3 – j 8.30 | 1.12 |  |
| Mmana-gal | 64.4 – j 3.18 | 0.8 |  |

# Neomom *.nml input file example

Hand edit or use neomom_input gui

&RunTitle    title = '40 m Delta Loop at 10 m' /

&Frequency_MHz   fmin  = 29.0   fmax  = 29.0   nFreq = 1/

&GROUND   Ground_Plane = real   epsilon = 14.0   sigma = 0.005/

&OPTONS   NBASISPERLAMBDA = 40/

&node_input   zHeight = 13.0   nNodes = 3   units = meters

node_list(1) = A, -10.0, 0.0, 0.0

node_list(2) = B, 10.0, 0.0, 0.0

node_list(3) = C, 0.0, -20.0, 0.0

/

&wire_primitive

tag   = W1,

nNodes = 4,

nodeTags = C B A C

radius     = 0.001

/

&excitation_input

wireTag = W1

nodeTag = C

voltage = 1.0

phase_deg = 0.0

/

# Neomom *.toml input file example

Possible alternative to namelist input.  Modern form for *.ini formats.  Widely supported.

[RunTitle] title = "40 m Delta Loop at 10 m"

[Frequency_MHz] fmin  = 29.0 fmax  = 29.0 nFreq = 1

[GROUND] Ground_Plane = "real" epsilon = 14.0 sigma = 0.005

[OPTIONS] NBASISPERLAMBDA = 40

[node_input] zHeight = 13.0 nNodes = 3 units = "meters"

[[node_input.node_list]] tag = "A" x = -10.0 y = 0.0 z = 0.0

[[node_input.node_list]] tag = "B" x = 10.0 y = 0.0 z = 0.0

[[node_input.node_list]] tag = "C" x = 0.0 y = -20.0 z = 0.0

[[wire_primitive]] tag = "W1" nNodes = 4 nodeTags = ["C", "B", "A", "C"] radius = 0.001

[excitation_input] wireTag = "W1" nodeTag = "C" voltage = 1.0 phase_deg = 0.0