neomom will be judged by ham's who compare results with mmana-gal and eznec. Input impedance will be one of the main comparison items.

Thought process

Thought process

This is the right lens to design around. Ham operators are pragmatic — they'll open all three codes on the same antenna and compare numbers. Understanding where agreement is tight versus where it diverges systematically will let you set honest expectations and, where possible, tune NeoMoM toward the NEC reference.

The three formulations side by side

|  | MMANA-GAL (MMINEC) | EZNEC / NEC-5 | NeoMoM |
| --- | --- | --- | --- |
| Basis functions | Pulse (constant per seg) | 3-term sinusoidal | Rooftop (triangular) |
| Testing | Point-matching at nodes | Galerkin (sinusoidal) | Galerkin (rooftop) |
| Source model | At node junction | Delta-gap at seg centre | Delta-gap at seg centre |
| Self-term | MMINEC near-field | Exact sinusoidal | Gibson analytical |
| Matrix symmetry | Non-symmetric | Symmetric | Symmetric |

NEC and NeoMoM are the closer pair — both Galerkin with delta-gap at segment centre. MMANA-GAL differs more fundamentally because the source sits at a node (current peak) rather than at a segment centre.

Where all three agree tightly

For resonant, well-meshed, centre-fed antennas the three formulations converge and hams will see agreement within a few ohms:

- Half-wave dipole in free space: NEC gives R≈73, X≈+42 Ω. All three should be within 2–3 Ω of each other at 20+ segments/λ.

- Quarter-wave monopole over a ground plane

- Yagi with centre-fed driven element

This is the "it just works" zone that builds trust.

Where they diverge and why

Off-resonance high-reactance antennas — EFHW at 2000–5000 Ω, short loaded verticals, top-hat loaded monopoles. Reactance is exquisitely sensitive to the self-term treatment and source placement. A 1 mm shift in effective feed location on a high-Q antenna can change X by hundreds of ohms. The MMINEC node-source vs NEC/NeoMoM segment-centre-source difference is the dominant error source here. Hams will see MMANA-GAL and NEC disagree by 20–50% in X for EFHW antennas — NeoMoM will track NEC if the source placement is matched.

Electrically short antennas — The MMINEC "end effect" is well known. MMANA-GAL systematically underestimates capacitive reactance on short antennas. NEC and NeoMoM are closer to the physical truth. Hams who have calibrated on MMANA-GAL will see NeoMoM report more negative X on a short vertical — NeoMoM is correct.

Junctions — The top-hat hub is where all three codes diverge most. MMANA-GAL handles it least rigorously (junction currents not strictly enforced). NEC-5 handles it well. NeoMoM with the anchor+partners basis is now correct but produces a different Z_in than NEC for off-resonance cases because the rooftop vs sinusoidal basis gives different effective current distributions near the hub.

Segment count sensitivity — MMINEC converges slowly and hams often run very coarse meshes (5–10 segments on a half-wave element). NEC converges faster. NeoMoM with Galerkin rooftop also converges fast. At coarse mesh density, MMANA-GAL will be the outlier.

Practical guidance for ham-facing validation

Run these three canonical cases and document the results:

- Half-wave dipole at a range of frequencies (0.5 to 30 MHz) — all three should agree within ±3 Ω in R and ±5 Ω in X. If NeoMoM passes this, it's credible.

- Quarter-wave monopole over a perfect ground, then over 4 / 8 / 16 radials — comparison with NEC-5 is the benchmark. Agreement within 5% in R establishes junction handling is correct.

- Folded dipole — known analytical result R≈292 Ω at resonance. All three should hit this within ±10 Ω. Tests the two-junction case.

- EFHW at 7 MHz (a common ham antenna) — NEC and NeoMoM should agree with each other. MMANA-GAL will likely differ. Document this explicitly and explain why rather than defending NeoMoM's number.

The most important message for hams

Frame it this way: NeoMoM uses the same mathematical formulation as NEC (Galerkin, delta-gap at segment centre, symmetric matrix). For resonant antennas it agrees with NEC/EZNEC to within 1–3%. For off-resonance, high-impedance antennas it agrees with NEC and disagrees with MMANA-GAL — and the NEC number is closer to what you measure on a vector network analyser. MMANA-GAL's MMINEC engine has known systematic errors for high-Z feeds that hams have learned to compensate for empirically; NeoMoM doesn't have those errors.

One concrete thing to implement now

Add a comparison table to NeoMoM's output for the standard half-wave dipole case — run it automatically as a self-test and print "NeoMoM: 73.1 + j42.3 Ω NEC reference: 73.1 + j42.1 Ω Δ: 0.0%/0.5%" at startup. Hams who see that will trust everything else that follows.